“Multi-Depth Adaptive Irrigation Controller with Root-Zone Proportional Regulation”

PROJECT REPORT

By

Joel Devasia – 23BCE2236

G Harshaditya Reddy – 23BCE0762

Winter 2025 - 2026

Course Code: BCSE305L

Name of the Course: Embedded Systems

Slot: G2+TG2

Name of the Faculty: Dr. NITHYA K

ABSTRACT

Water management is one of the biggest challenges in agriculture, especially when traditional irrigation systems still depend on fixed schedules or moisture readings taken from only one level of soil. In many cases, these methods do not show the actual amount of water available to the roots, which is the most important part of the plant for water absorption. Because of this, plants may receive too much or too little water, leading to poor growth and unnecessary wastage of water. This project introduces a multi-depth adaptive irrigation controller that is designed to make irrigation more accurate and efficient by using real-time data from different soil depths.

The traditional irrigation techniques are constant in nature and depend upon fixed timings or moisture measurement of the soil at a single location of the soil profile which might not be an indicator of the actual moisture content present in the soil available for the plant roots. In contrast, the proposed system utilizes a depth-based adaptive irrigation controller to measure the moisture content in the soil at various depths including the root zone, as well as humidity and lighting conditions. By analysing the data, the irrigation process is controlled automatically depending upon the requirement of the plant.

OBJECTIVE

To design a multi-layered soil moisture sensing system capable of distinguishing between surface and root-area fundamental moisture assessments for correct determination of water levels in relation to each plant.

To create a mechanism for assessing vertical moisture gradients to analyze the distribution of water in the soil at various depths.

To build an embedded irrigation controller that adjusts irrigation duration in response to real-time deficits in root-area moisture.

To integrate environmental factors such as temperature, humidity and light intensity to calculate overall metric on the plant condition.

To develop a closed-loop, hardware-based adaptive irrigation control system aimed at enhancing water-use efficiency and optimizing hydration in the root zone.

CHAPTER 1

INTRODUCTION

The project comes under the domain of embedded systems and agriculture, specifically focusing on irrigation management and control systems that utilizes multi-depth soil moisture sensing along with the integration of adaptive environmental parameters. The details of this invention involve a hardware-based, depth-sensitive irrigation controller module that assesses/samples soil moisture across various soil layers to identify surface and root-zone moisture conditions and adjusting irrigation plans dynamically based on this obtained information and environmental variables (such as temperature, humidity, light intensity etc.).  These values and variables are updated constantly at regular intervals.

Traditional irrigation systems rely on either single-depth soil moisture sensing or fixed watering schedules. While there are systems that can automate this process, the majority still operate on a similar scale of surface moisture versus hydration of the root zone. These systems fail to differentiate between surface moisture and the hydration of the root zone, which often leads to overwatering or not delivering enough water to the roots altogether. While some advanced irrigation technologies consider environmental factors such as temperature and humidity, they typically rely on predictive models or separate parameter assessments and do not provide real-time analysis of vertical soil moisture. Current systems do not adjust irrigation dynamically based on the measured moisture deficit in the root zone.

The invention solves these problems with a multi-depth soil moisture sensing system suitable for measuring surface and root-zone moisture while measuring the vertical moisture gradient.

An integrated controller combines environmental factors, including temperature, humidity, and light intensity, to generate an overall metric of plant health.

Utilizing real-time data on root-zone moisture deficit and environmental conditions, the system adjusts irrigation duration dynamically through an adaptive control mechanism instead of relying on fixed time intervals. An integrated embedded design developed and integrated multi-layer soil profiling, root-zone deficit based proportional irrigation control and integration of environmental parameters in a closed-loop hardware aimed at water-my-use efficiency leading to improvement in hydration at root level.

CHAPTER 2

LITERATURE SURVEY

Prior patents:

| Patent / Publication Year | Core Innovation / Key Description | Technical Limitations / Gaps Identified |
| --- | --- | --- |
| US 8,443,225 B2 (2013) | An automatic irrigation controller that activates water pumps using soil moisture threshold analysis. It relies on basic sensor feedback to trigger irrigation when moisture drops below a preset level. | Uses single-depth sensing, unable to differentiate between surface and root-zone moisture. This can lead to inefficient watering (e.g., over-irrigation when only the surface is dry). Additionally, the fixed threshold-based control logic lacks adaptability to changing soil conditions or crop requirements. |
| US 9,420,123 B2 (2016) | Irrigation scheduling based on environmental parameters such as temperature and evapotranspiration (ET) to estimate crop water needs. | Relies entirely on predictive climatic models instead of real-time soil data, leading to inaccurate scheduling. Lacks real-time root-zone sensing and ignores vertical soil moisture variability, reducing precision. |
| WO 2017/089321 A1 (2017) | Describes an IoT-enabled smart irrigation system that allows remote monitoring and control of irrigation processes using soil moisture data. | Although IoT-enabled, the system uses only a single moisture probe, which does not capture soil moisture variability across depths. Furthermore, irrigation is not proportionally controlled based on moisture deficit, leading to non-optimized water usage. |
| US 10,321,456 B2 (2019) | Presents an automated irrigation system with programmable watering intervals and schedules for agricultural fields. | The system primarily relies on time-based or preset scheduling, rather than real-time feedback. It lacks adaptive decision-making based on soil moisture conditions, especially across multiple depths, making it less efficient under dynamic environmental conditions. |
| CN 109876543 A (2019) | Integrates environmental sensors (such as temperature, humidity, etc.) into irrigation systems to enhance decision-making. | Environmental parameters are analyzed independently, leading to fragmented decisions. Lacks integrated plant condition scoring and depth-aware moisture control, limiting precision irrigation capabilities. |
| US 11,234,567 B2 (2021) | Multi-sensor agricultural monitoring platform combining environmental and soil parameters for precision farming insights. | Primarily a monitoring-focused system without closed-loop irrigation control. Does not translate sensor data into automated adaptive irrigation actions based on root-zone moisture deficits. |
| EP 3 456 789 A1 (2022) | Integrates soil moisture sensing with weather data analytics for improved irrigation decision-making using a data-driven approach. | Provides analytical insights only, lacking real-time embedded irrigation control. No mechanism for dynamic adjustment of irrigation duration based on actual moisture deficits across different soil depths. |

Prior literature papers:

| Literature / Publication Year | Core Innovation / Key Description | Technical Limitations / Gaps Identified |
| --- | --- | --- |
| IEEE Sensors Journal (2018) | Proposes a wireless sensor network (WSN)-based architecture designed for continuous soil moisture monitoring across agricultural fields. The system enables remote data acquisition, real-time transmission, and provides basic automation support for irrigation awareness. | Primarily a monitoring-focused system that emphasizes data collection rather than control. It lacks a closed-loop irrigation mechanism capable of dynamically adjusting irrigation based on soil needs. Additionally, it does not support multi-depth soil moisture sensing, limiting accurate assessment of root-zone water availability. |
| Computers and Electronics in Agriculture (2019) | Develops evapotranspiration (ET)-based irrigation scheduling models using climatic variables such as temperature, relative humidity, and solar radiation to estimate crop water requirements. | A model-based approach that relies on predicted climatic conditions instead of real-time soil sensor data, leading to potential inaccuracies. It lacks vertical soil moisture profiling and does not support adaptive real-time irrigation control, reducing precision. |
| Agricultural Water Management (2020) | Examines soil moisture threshold-based irrigation strategies aimed at improving water conservation and optimizing irrigation efficiency. Irrigation is triggered when moisture falls below predefined threshold levels | Uses single-depth soil moisture sensing, which does not accurately represent root-zone conditions. The irrigation logic is static and threshold-based, lacking dynamic or proportional control to handle varying moisture deficits across different soil layers. |
| IEEE Access (2021) | Presents an IoT-based smart farming framework integrating multiple environmental sensors (temperature, humidity, etc.) for enhanced agricultural monitoring and system awareness. | Although multiple environmental parameters are integrated, the system lacks dedicated irrigation control strategies focused on root-zone moisture conditions. It also does not implement real-time embedded decision-making for automated and adaptive irrigation execution. |
| Sensors (MDPI) (2021) | Introduces a multi-sensor system for monitoring crop health, including environmental and soil-related parameters. | Primarily designed for monitoring and visualization, without incorporating automated irrigation actuation mechanisms. It does not utilize multi-layer soil moisture data to guide irrigation decisions, limiting its application in precision irrigation. |
| Journal of Hydrology (2022) | Focuses on analyzing soil water availability and estimating crop water requirements using hydrological modeling and simulation techniques. | A theoretical and simulation-based approach without implementation as a real-time embedded control system. It lacks hardware integration and adaptive irrigation control mechanisms based on actual field conditions. |
| Computers and Electronics in Agriculture (2023) | Explores machine learning-based irrigation optimization techniques aimed at improving water-use efficiency using data-driven approaches. | Predominantly software- and data-driven, with limited emphasis on hardware-level sensing accuracy. It does not adequately address depth-aware soil moisture control or real-time embedded irrigation execution, which are essential for practical agricultural deployment. |

CHAPTER 3

SYSTEM OVERVIEW

The system includes a multi-depth soil sensing module, an environmental sensing module, an embedded control unit and irrigation actuation mechanism. The combination of these systems creates a closed-loop adaptive irrigation system where the water delivery is adjusted based on root-zone moisture and environmental conditions

The invention uses a multi-depth soil moisture sensing system suitable for measuring surface and root-zone moisture while measuring the vertical moisture gradient.

An integrated controller combines environmental factors, including temperature, humidity, and light intensity, to generate an overall metric of plant health.

Utilizing real-time data on root-zone moisture deficit and environmental conditions, the system adjusts irrigation duration dynamically through an adaptive control mechanism instead of relying on fixed time intervals. An integrated embedded design developed and integrated multi-layer soil profiling, root-zone deficit based proportional irrigation control and integration of environmental parameters in a closed-loop hardware aimed at water-my-use efficiency leading to improvement in hydration at root level.

CHAPTER 4

HARDWARE DESIGN

The system includes a multi-depth soil sensing module, an environmental sensing module, an embedded control unit and irrigation actuation mechanism. The combination of these systems creates a closed-loop adaptive irrigation system where the water delivery is adjusted based on root-zone moisture and environmental conditions.

The multi-depth soil sensing module features at least two moisture sensors located at various vertical depths within the soil profile: one sensor at the surface level and another in the root zone. These sensors produce electrical signals that are proportional to the volumetric water content, which are then relayed to the embedded controller via either analog or digital interfaces.

It consists of integrated sensors T, RH and light intensity. These sensors provide data in real time on the environmental situation that influences plant water needs. The embedded controller uses these signals to produce a composite metric of plant condition.

This embedded control unit programming deals with the operation of signal acquisition, analog-to-digital conversion (if necessary), filtration of data, determining vertical moisture differentials and root-zone deficits. The controller performs a proportional adaptive control algorithm to determine irrigation duration depending on the difference between actual root-zone moisture and an optimal threshold that can be adapted according to environmental conditions.

An electrically driven pump is the actuation mechanism, or an electric solenoid valve controlled by a relay or transistor interface circuit. After calculating the necessary duration of irrigation, the controller triggers the actuating system for the desired interval.

The system persistently observes soil moisture levels following irrigation and adjusts control decisions instantaneously, thus establishing a closed-loop feedback mechanism. This arrangement guarantees that water distribution corresponds with the genuine absorption conditions of plants, rather than merely surface moisture readings or predetermined timing schedules.

The establishment of multi-layer soil profiling, provides layers of environmental parameter fusion as a new perturbation in the embedded proportional irrigation control to shape a digital watershed irrigation even, thus providing the basis for wise irrigation aimed at improving efficient water-use and reducing over-irrigation whilst refining root-zone hydration precision.

CHAPTER 5

IMPLEMENTATION AND RESULTS

- Result – 1 (Normal Irrigation i.e High Hydration Mode)

------ Greenhouse Status ------

Profile: High Hydration

Temp: 27.4 C

Humidity: 68 %

Surface Moisture: 40 %

Root Moisture: 52 %

Moisture Gradient: 12

Light: 75 %

PCI: 72.5

Condition: Good

-------------------------------

Irrigation Duration (ms): 2000

- Result – 2 (Rain Detected)

------ Greenhouse Status ------

Profile: Balanced Growth

Temp: 24.8 C

Humidity: 82 %

Surface Moisture: 70 %

Root Moisture: 75 %

Moisture Gradient: 5

Light: 40 %

PCI: 65.8

Condition: Good

-------------------------------

- Result – 3 (Dry Soil)

------ Greenhouse Status ------

Profile: Low Hydration

Temp: 32.1 C

Humidity: 45 %

Surface Moisture: 20 %

Root Moisture: 25 %

Moisture Gradient: 5

Light: 90 %

PCI: 48.2

Condition: Critical

-------------------------------

Irrigation Duration (ms): 5000

- Result – 4 (Ideal Conditions)

------ Greenhouse Status ------

Profile: Balanced Growth

Temp: 25.2 C

Humidity: 75 %

Surface Moisture: 78 %

Root Moisture: 82 %

Moisture Gradient: 4

Light: 65 %

PCI: 84.3

Condition: Excellent

-------------------------------

Irrigation Duration (ms): 0

- Result – 5 (High Temperature Impact)

------ Greenhouse Status ------

Profile: High Hydration

Temp: 38.5 C

Humidity: 50 %

Surface Moisture: 55 %

Root Moisture: 60 %

Moisture Gradient: 5

Light: 85 %

PCI: 55.6

Condition: Moderate

-------------------------------

Irrigation Duration (ms): 800

SAMPLE CODE

#include <LiquidCrystal.h>

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

// -------- PIN DEFINITIONS --------

#define S1 A0

#define S2 A1

#define S3 A2

#define TEMP A3

#define RAIN 7

#define MODE_BTN 6

#define RELAY 8

#define LED_PUMP 9

#define LED_NORMAL 10

#define LED_RAIN 13

int mode = 0;

// -------- PROFILE PARAMETERS --------

float threshold[3] = {75.0, 60.0, 45.0};

// Root importance changes per profile

float rootWeight[3] = {0.8, 0.7, 0.6};

float surfaceWeight[3] = {0.2, 0.3, 0.4};

void setup() {

Serial.begin(9600);

pinMode(RAIN, INPUT_PULLUP);

pinMode(MODE_BTN, INPUT_PULLUP);

pinMode(RELAY, OUTPUT);

pinMode(LED_PUMP, OUTPUT);

pinMode(LED_NORMAL, OUTPUT);

pinMode(LED_RAIN, OUTPUT);

lcd.begin(16, 2);

lcd.print("Greenhouse Sys");

delay(2000);

lcd.clear();

randomSeed(analogRead(0));

}

void loop() {

checkModeButton();

// -------- SENSOR READINGS --------

float surfaceMoisture = readMoisture(S1);

float midMoisture = readMoisture(S2);

float deepMoisture = readMoisture(S3);

float temp = readTemperature();

float rootMoisture = (0.4 * midMoisture) +

(0.6 * deepMoisture);

float RZWI = (surfaceWeight[mode] * surfaceMoisture) +

(rootWeight[mode] * rootMoisture);

float moistureGradient = abs(rootMoisture - surfaceMoisture);

float humidity = random(40, 90);

float light = random(10, 100);

float tempFactor = 100 - abs(25 - temp);

float PCI = (0.4 * RZWI) +

(0.2 * humidity) +

(0.2 * light) +

(0.2 * tempFactor);

bool rainDetected = (digitalRead(RAIN) == LOW);

// -------- DYNAMIC IRRIGATION CONTROL --------

if (rainDetected) {

digitalWrite(RELAY, LOW);

digitalWrite(LED_RAIN, HIGH);

digitalWrite(LED_PUMP, LOW);

digitalWrite(LED_NORMAL, LOW);

}

else {

digitalWrite(LED_RAIN, LOW);

// Root moisture deficit calculation

float deficit = threshold[mode] - rootMoisture;

if (deficit < 0)

deficit = 0;

// Base irrigation duration proportional to deficit

int irrigationTime = deficit * 80;

// Add adjustment based on stratification intensity

irrigationTime += moistureGradient * 40;

// Limit maximum irrigation time (safety cap)

if (irrigationTime > 5000)

irrigationTime = 5000;

if (irrigationTime > 0) {

digitalWrite(RELAY, HIGH);

digitalWrite(LED_PUMP, HIGH);

digitalWrite(LED_NORMAL, LOW);

delay(irrigationTime);

digitalWrite(RELAY, LOW);

digitalWrite(LED_PUMP, LOW);

}

else {

digitalWrite(RELAY, LOW);

digitalWrite(LED_PUMP, LOW);

digitalWrite(LED_NORMAL, HIGH);

}

Serial.print("Irrigation Duration (ms): ");

Serial.println(irrigationTime);

}

String condition = classifyCondition(PCI);

String profile = getProfileName(mode);

displayLCD(PCI, profile);

displaySerial(temp, humidity, surfaceMoisture,

rootMoisture, light, PCI,

moistureGradient, condition,

profile);

delay(2000);

}

// -------- FUNCTIONS --------

float readMoisture(int pin) {

int value = analogRead(pin);

return map(value, 1023, 0, 0, 100);

}

float readTemperature() {

int val = analogRead(TEMP);

float voltage = val * (5.0 / 1023.0);

return voltage * 100;

}

void checkModeButton() {

static bool lastState = HIGH;

bool currentState = digitalRead(MODE_BTN);

if (lastState == HIGH && currentState == LOW) {

mode++;

if (mode > 2) mode = 0;

delay(300);

}

lastState = currentState;

}

String classifyCondition(float pci) {

if (pci > 80) return "Excellent";

if (pci > 65) return "Good";

if (pci > 50) return "Moderate";

return "Critical";

}

String getProfileName(int mode) {

if (mode == 0)

return "High Hydration";

if (mode == 1)

return "Balanced Growth";

return "Low Hydration";

}

void displayLCD(float pci, String profile) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(profile);

lcd.setCursor(0, 1);

lcd.print("PCI:");

lcd.print(pci, 1);

}

void displaySerial(float temp,

float humidity,

float surface,

float root,

float light,

float pci,

float gradient,

String condition,

String profile) {

Serial.println("------ Greenhouse Status ------");

Serial.print("Profile: ");

Serial.println(profile);

Serial.print("Temp: ");

Serial.print(temp);

Serial.println(" C");

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.println(" %");

Serial.print("Surface Moisture: ");

Serial.print(surface);

Serial.println(" %");

Serial.print("Root Moisture: ");

Serial.print(root);

Serial.println(" %");

Serial.print("Moisture Gradient: ");

Serial.println(gradient);

Serial.print("Light: ");

Serial.print(light);

Serial.println(" %");

Serial.print("PCI: ");

Serial.println(pci);

Serial.print("Condition: ");

Serial.println(condition);

Serial.println("-------------------------------");

Serial.println();

}

CONCLUSION

To conclude, the invention’s adaptive irrigation system gives much more effective approach to irrigation than normal irrigation systems by focusing on the root area moisture instead of only depending on the surface readings on a static place or recurring timeframes. By combining all the soil moisture data on all depth levels with all the environmental factors taken into account , the system can do better irrigation decisions in real time. These aspects help reduce water wastage and support better plant health.

Overall the invention combines aspects of multi-layer soil detection and environmental factors and real time control into single embedded system. It is practical to use and useful to implement in irrigation field. The system can help save and be more efficient in resources.

PROJECT VIDEO LINK

Link of project demonstration Video:    https://youtu.be/gkhZAeOm4Kg?si=qyh3VJ5NThnNEVpC